| UEW Univ. of Education, Winneba & Kayaba Labs | Securing the Digital Mine with a Metaheuristic-Optimized Deep Learning Adaptive System John Okyere, Ezekeil Baah, Clement Baffour, Parker Paa Annobil, George Akwesi Bonnah Department of ICT, University of Education, Winneba & Kayaba Labs ¦ Saint Petersburg Mining University 2026 | UNESCO Russian-African Forum Track 3: Smart Subsoil |
| --- | --- | --- |

| 1. Industrial Threat Landscape • Mining 4.0: African/Russian mines deploy IoT & SCADA telemetry to maximize ore yield. • Air-Gap Collapse: Connecting OT to cloud twins exposes Modbus/DNP3 protocols to zero-day attacks. • Downtime Losses: Unplanned downtime costs USD $50k-$500k/hr; gas/ventilation disruption endangers lives. • IT IDS Mismatch: Traditional tools take 150+ ms, violating the 20-50ms SCADA control loop deadline. 2. Method & BWOA Mathematics • Data Ingestion: Sniffer agent (@mhiskall282/unesco-mine-sec-cli) parses Modbus/TCP frames. • Shrinking Encircling: D = ¦C*X* - X¦, X(t+1) = X* - A*D (A = 2a*r1 - a, C = 2*r2). • Spiral Bubble-Net: X(t+1) = D'*exp(b*l)*cos(2*pi*l) + X* (b=1.0, l in [-1, 1]). • V-Shaped Transfer: V(x) = ¦x / sqrt(1 + x^2)¦, stochastic bit-flipping on velocity. • Accuracy Floor Fitness: Fit(X) = 0.3*(1 - Acc) + 0.7*(¦X¦/D) + Penalty (Acc >= 75%). • Float16 Quantization: Model compressed to 0.82 MB for sub-millisecond execution on 1GB RAM edge gateways. | 3. 4-Layer Architecture Pipeline • Decoupled 4-layer edge architecture ensures sub-millisecond real-time threat evaluation: 4. BWOA Optimization Analysis • BWOA convergence reached at iteration 23/100, maintaining 92.31% RF cross-validation accuracy. • Selected 10 Features: src_bytes, service, flag, serror_rate, same_srv_rate, diff_srv_rate, dst_host_diff_srv_rate, protocol_type, hot, su_attempted. | 5. Empirical Hardware Latency Profile • Single-sample inference latency benchmarked across hardware platforms vs 100ms SCADA ceiling: 6. Multi-Class Confusion Matrix • Held-out test set evaluation (22,544 samples) on 10 BWOA-selected features: 7. Conclusions & UN SDG Impact • 0.76ms Edge Latency: 207x faster than baseline on Raspberry Pi 4B (1GB RAM), passing SCADA constraints. • High Detection Fidelity: 70.56% multi-class accuracy, 96.89% benign precision, 89.04% DoS recall. • Industrial ROI: 200x-300x return averting $50k-$500k/hr outages while protecting miner lives (SDG 8 & 9). 8. References • 1. Alanazi, M., et al. (2022). SCADA vulnerabilities & attacks. Computers & Security, 125, 103028. • 2. Almomani, O., et al. (2025). SCADA intrusion detection in IIoT. Symmetry, 17(4), 480. • 3. Mirjalili, S., & Lewis, A. (2016). Whale optimization algorithm. Adv. Eng. Software, 95, 51-67. • 4. Minerals Commission of Ghana. (2024). Digital telemetry & cybersecurity compliance policy. |
| --- | --- | --- |